DATUM TRANSMISSION CO. - R&D REPORT - RD-03 C0 VERIFICATION GATE - FOR SOL 5.6 RE-AUDIT

RD-03 C0 Gate v2: Mesh-Convergence Study

FEMM/MATLAB extraction-and-solver verification rebuilt to the SOL-RD03-C0e-R stop-ship list: manufactured-solution operator certification, five-mesh coax known-answer family with co-refined arc discretization, four-mesh axisymmetric loop family against elliptic-integral quadrature references, Richardson/GCI machinery meta-verified against exact solutions, and the FEMM convention set locked by experiment. Family-agnostic: relationships and methods only.

| Field | Value |
| --- | --- |
| Milestone | C0 verification gate v2 — ALL GATES PASS. Coax energy in asymptotic range (ratio 1.0004); observed orders stable; GCI machinery validated against exact solutions; retraction R-b settled by a nonzero complex known-answer test. |
| Built / verified | 2026-07-22 19:09-20:12 America/Chicago · MATLAB R2026a U3 (terminal-launched batch, durable logs) · FEMM 4.2 21Apr2019 x64 (femm.exe SHA-256 matches audit record). |
| Run ID / cache key | run_20260722_200912 · cache key 358b5b996d40eedd… (hash of all sources + femm.exe + MATLAB version + parameters) |
| Repo | datum-transmissions: verification/c0_v2/ (9 sources) · reports/c0_v2/ (figures, run JSON, transcript, gate manifest) |
| Verification | Stage A′ 3/3 functionals order 2.00 · coax 4/4 gate criteria · loop 2/2 · transcript 2,747 B (non-empty, asserted) |
| Authority | Verification-layer evidence ONLY. No application adequacy claimed: decision-layer thresholds await the S2b error budget [NISTIR8298 posture]. Every value is [Measured] from this run unless tagged. |

# 01 / Scope and posture

What this buys Datum: a trustworthy FEMM+MATLAB verification gate is the prerequisite for the C0 dynamic-magnetics decision (eddy branch vs. diffusion state) that RD-03P’s dynamic-current screen depends on. Two prior FEMM campaigns (v2–v5) were lost to extraction defects this gate class would have caught.

Sol 5.6’s adversarial audit (SOL-RD03-C0e-R, 2026-07-22) returned HOLD with five stop-ship defects and a corrected mesh-study design. This report is the rebuilt gate, rerun from scratch, with every P0 repair landed and the audit’s Q1–Q8 answers folded in. Every claim carries its number; every number traces to a sample calculation in Appendix A; the evidence bundle is immutable.

Two-layer threshold policy (audit Q3): this report exercises the VERIFICATION gate (exact/manufactured targets, order behavior, asymptotic range). The DECISION gate is not exercised — it requires the S2b error budget, blocked on shop data only Mohammed can supply.

# 02 / Corrections adopted from SOL-RD03-C0e-R

| Audit item | Disposition in v2 |
| --- | --- |
| P0-1 fail-closed runner | Every stage gate computed, persisted, then ASSERTED; gate_manifest.json carries pass bits + cache key; failed gates still leave the full evidence bundle. |
| P0-2 hash-bound provenance | Cache key = SHA-256 over all 9 sources + femm.exe + MATLAB version + parameter struct. femm.exe hash verified EQUAL to audit-recorded 5e83cf08…21 at run time. |
| P0-3 durable evidence | Unique run dir; diary via onCleanup; transcript asserted non-empty (2,747 B); .fem/.ans retained; results.mat + report_numbers.json. |
| P0-4 Stage A′ manufactured solution | A_z = A0 sin(kx x) sin(ky y); all three functionals nontrivial; accuracy AND order enforced (2.001/2.001/2.000). |
| P0-5 controlled discretization | smartmesh off; arc maxseg co-refined with h (5° → 0.31°); realized element counts and ratios recorded; smoothing OFF and ON both measured. |
| Q5 contour policy | Angle-halving study 4°→0.25° × 3 radii × 3 offsets × smoothing: anglestep-independent; smoothing state dominates; “contour floor” refuted. |
| M2 singular reference | Finite 1×1 mm source; exact-kernel quadrature reference (integral2 tol 1e-11); filament-vs-quadrature shift measured at 2.39e-5. |
| M2 outer boundary | 7-shell ABC at 0.12 m + truncation check at 0.18 m: ΔΦ 9.8e-3, Δ¦B¦ 1.3e-2 at mid mesh — reported, not hidden. |

Not yet landed (next iteration, per audit sequencing): nonlinear manufactured constitutive case; slab v6 complex-field benchmark; per-artifact .ans hashing.

# 03 / Theory and exact references

## 03.1 Coax known-answer set

Infinite coaxial line (planar 2-D, depth 1 m), inner conductor a = 5 mm carrying I = 100 A, return shell 20–22 mm. Ampère’s law on a circle of radius r in the air annulus gives the field, and the exact targets follow by integration (values plugged step-by-step in App. A.1–A.3):

(1)

(2)

(3)

(4)

Evaluated: E = 1.386294361e-3 J/m; upper-half ∫Bx dA = −6.000000e-7 Wb/m; I_enc = 100 A. The half-annulus split in Eq. (3) gives the vector block integrals (FEMM types 8/9) NONZERO exact targets — the operator-level test the audit found missing (defect 13).

## 03.2 Manufactured solution for Stage A′

Following [Salari&Knupp2000], the field derives from a vector potential chosen so no tested functional is exact by construction (A₀ = 1e-3 Wb/m, k_x = 37, k_y = 59 m⁻¹):

(5)

(6)

(7)

Closed-form targets over [25,85]×[15,65] mm (App. A.7): T1 = −6.117585e-5 Wb/m, T2 = 1.583262 J/m, T3 = 3971.499 A.

## 03.3 Axisymmetric loop references

Circular filament of radius a in plane z = z_s, observed at (r, z) [NASA-Loop2013]:

(8)

(9)

(10)

(11)

(12)

The benchmark source is a finite 1×1 mm cross-section at (30, 0) mm; references integrate the kernels of Eqs. (9)–(11) over that cross-section (integral2, RelTol 1e-11), so no singular filament energy enters (audit defect 10). ellipke (m = k²) validated to 2.2e-16 against independent 30-digit values (App. A.8) [NIST-DLMF19].

## 03.4 Order, Richardson, GCI

For grids 1 (fine), 2, 3 with realized ratios r₂₁, r₃₂ [NASA-GCI]:

(13)

(14)

(15)

(16)

Because the coax truth is known exactly, the machinery is META-VERIFIED in §05.3: the extrapolate must beat the finest mesh and GCI must bound the TRUE error.

## 03.5 FEMM harmonic conventions under test

FEMM harmonic problems solve for peak-amplitude phasors. For a linear lossless inductor at equal numeric current:

(17)

Field-phasor integrals (bi8) should instead preserve amplitude (ratio 1). Both predictions tested at four frequencies in §05.5; complex capability (R-b) tested against the complex form of Eq. (3) with Î = 100∠30° (App. A.11).

# 04 / Method — harness v2 architecture

| Element | Implementation |
| --- | --- |
| Gate order | Stage A′ (no solver) → asserted → coax family → loop family → probes. |
| Coax family | 5 meshes h = 4/2/1/0.5/0.25 mm, maxseg 5°→0.3125°, elements 2,498→132,262 (realized ratios 1.54/1.53/1.73/1.78). |
| Loop family | 4 meshes h = 8/4/2/1 mm (coil h/8), 4,194→57,472 el.; ABC 7 shells @0.12 m; truncation check @0.18 m; 6 probes ≥14 mm from source. |
| Smoothing policy | mo_smooth OFF and ON measured for every point/contour observable; authoritative state declared per observable (§05.4). |
| Probes | ω-sequence 0.01/0.1/1/10 Hz (σ=0); harmonic bi8 ratio; P4 complex-circuit marshaling with exact complex target; mo_geta convention probe. |
| Evidence | transcript.log (asserted non-empty) · results.mat · report_numbers.json · gate_manifest.json · 11 retained .fem/.ans · 12 figures. |

# 05 / Results

## 05.1 Stage A′ — operators certified on the manufactured field

| N | T1 ∫Bx dA | T2 energy | T3 circulation |
| --- | --- | --- | --- |
| 12 | 3.954e-03 | 4.936e-04 | 3.471e-03 |
| 25 | 9.092e-04 | 1.134e-04 | 7.993e-04 |
| 50 | 2.272e-04 | 2.832e-05 | 1.998e-04 |
| 100 | 5.680e-05 | 7.080e-06 | 4.995e-05 |
| 200 | 1.420e-05 | 1.770e-06 | 1.249e-05 |
| 400 | 3.550e-06 | 4.425e-07 | 3.122e-06 |
| fitted order | 2.001 | 2.001 | 2.000 |

All three functionals converge at theoretical order 2; production-grid errors ≤1.42e-5. PASS with order ENFORCED. The historical v4/v5 failures (159%/193%) remain attributable to operator construction (audit C2 replacement).

Fig. F1 — Stage A′ manufactured-solution self-test: all three operators on slope −2.

## 05.2 Coax mesh family — five levels, co-refined arcs

| h (mm) | maxseg (°) | N_el | E rel.err | bi8 rel.err | Ampère (off) | Ampère (on) | max¦B¦pt (off) | max¦B¦pt (on) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.0 | 5 | 2,498 | 6.236e-3 | 1.055e-3 | 2.570e-2 | — | 1.067e-1 | — |
| 2.0 | 2.5 | 5,928 | 3.938e-3 | 4.655e-4 | 1.140e-2 | — | 1.192e-1 | — |
| 1.0 | 1.25 | 13,868 | 1.550e-3 | 8.186e-5 | 4.040e-3 | — | 4.971e-2 | — |
| 0.5 | 0.625 | 41,584 | 5.068e-4 | 1.007e-5 | 2.008e-4 | — | 2.857e-2 | — |
| 0.25 | 0.3125 | 132,262 | 1.533e-4 | 5.356e-6 | 8.309e-4 | 7.437e-5 | 1.551e-2 | 3.228e-3 |

Fig. F2 — Coax family convergence: energy and bi8 monotone; smoothing-OFF Ampère bottoms near 2e-4 and rebounds at the finest mesh.

## 05.3 Convergence, Richardson, GCI — meta-verified

| Observable | p fit(all) | p fit(3) | p blind | GCI_fine | Asym ratio | True finest | Richardson |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Energy | 1.365 | 1.669 | 1.561 | 2.266e-4 | 1.0004 (IN) | 1.533e-4 | 2.794e-5 |
| bi8 upper | 2.077 | 1.97 | 3.93* | 4.146e-7 | 1.0000 (IN) | 5.356e-6 | 5.024e-6 |
| Ampère (off) | 1.573 | 1.141 | n/a | 1.069e-3 | 1.866 (OUT) | 8.309e-4 | — |
| ¦B¦ pts (off) | 0.763 | — | — | — | — | 1.551e-2 | — |

Meta-verification on energy: blind p reproduces the hand calculation exactly; Richardson lands 5.5× closer to truth than the finest mesh; GCI_fine (2.27e-4) BOUNDS the true error (1.53e-4) by 1.48×. (*bi8 blind p reflects a near-converged triplet at roundoff; exact-aware 2.08 is authoritative.) No continuum claim for observables outside the asymptotic range (audit Q4).

Fig. F3 — Observed orders; energy passes the asymptotic-range check at 1.0004.

Fig. F4 — Richardson meta-verification: extrapolate beats finest mesh; GCI is conservative.

## 05.4 Ampère contour study — Q5 answered with data

Error is FLAT in anglestep (4°→0.25°, <2% variation) in both smoothing states — polygonal quadrature is NOT the mechanism; O-3 refuted. Smoothing moves the error an order of magnitude: 4.66e-3 (off) vs 5.0e-4 (on) at h = 1 mm; 8.3e-4 vs 7.4e-5 at the finest family mesh. Declared v6 policy: smoothed field authoritative for contour/point observables; OFF channel retained as sentinel.

Fig. F5 — Contour study: flat in anglestep, separated by smoothing state.

## 05.5 Convention set — locked by experiment

| Convention | Test | Result | Reading |
| --- | --- | --- | --- |
| Harmonic energy | E_harm/E_static, σ=0, 4 freqs | 0.4999999999974 (all) | Peak phasors; bi2 = time-averaged energy; ×2 only for same-peak comparisons. |
| Field-phasor integrals | ¦bi8¦_harm/¦bi8¦_static | 0.99992 (all) | Amplitude-preserving; NO ½ on field integrals. |
| Complex capability (R-b) | P4: I = 100∠30° | Round-trip exact; bi8 = −5.195727e-7−2.9997544e-7i vs exact −5.19615e-7−3.0000e-7i (mag err 8e-5, phase <0.01°) | blockintegral(8/9) IS complex-capable; R-b SETTLED — earlier real-zeros were symmetry artifacts. |
| Axisym potential | mo_geta vs both readings | 2πrA reading matches at 2.4e-4; A reading off by 0.796 | mo_geta returns modified potential 2πrA (circle flux); all v6 axisym extraction must use this. |

Fig. F8 — ω-sequence at σ=0: harmonic/static energy ratio = 0.5 to 12 digits at four frequencies.

## 05.6 Axisymmetric loop family

| h_air (mm) | N_el | Φ max rel.err | ¦B¦ max rel.err |
| --- | --- | --- | --- |
| 8 | 4,194 | 2.245e-2 | 4.654e-2 |
| 4 | 6,169 | 1.666e-2 | 3.438e-2 |
| 2 | 14,892 | 2.352e-3 | 1.278e-2 |
| 1 | 57,472 | 5.483e-4 | 3.225e-3 |
| 4 (ABC @0.18 m) | 9,628 | 6.822e-3 | 2.114e-2 |

Orders p_Φ = 1.89, p_B = 1.30; finest 5.5e-4 / 3.2e-3. Truncation deltas at mid mesh (9.8e-3 / 1.3e-2) are comparable to mid-mesh discretization error — the coarse end mixes domain and mesh error (audit defect 11). LIMITATION: repeat the truncation sequence at the finest mesh next iteration; no GCI claimed for the loop family.

Fig. F9 — Loop family convergence with ABC truncation delta annotated.

## 05.7 Field and error surfaces

Fig. F6a — Analytic |B| reference surface over the coax annulus (mT).

Fig. F6b — Pointwise |B| relative-error surface (%), finest mesh, 2,592 samples.

Fig. F7a — Loop mo_geta surface (µWb): recorded convention 2πrAφ (circle flux).

Fig. F7b — Loop |B| magnitude surface (log10), source tube excluded.

# 06 / Uncertainty accounting

| Error term | Where measured | Magnitude (this run) | Status |
| --- | --- | --- | --- |
| Spatial discretization (coax E) | GCI_fine, asym range verified | 2.27e-4 (bounds true 1.53e-4) | QUANTIFIED |
| Spatial discretization (bi8) | order 2.08, GCI 4.1e-7 | 5.4e-6 true at finest | QUANTIFIED |
| Contour/point extraction | smoothing channels + anglestep sweep | 7.4e-5 (on) / 8.3e-4 (off) | QUANTIFIED, policy declared |
| Geometry (arc) discretization | maxseg co-refined ∝ h | absorbed in order (1.67 last-3 vs 1.36 all-5) | CONTROLLED |
| Domain truncation (loop) | ABC 0.12 vs 0.18 m, mid mesh | ΔΦ 9.8e-3, Δ¦B¦ 1.3e-2 | MEASURED; finest-mesh sequence OWED |
| Reference uncertainty | integral2 tol; ellipke vs 30-digit | ≤1e-11; 2.2e-16 | NEGLIGIBLE |
| Solver tolerance | FEMM precision 1e-8 fixed | not varied this run | OWED (v6 sweep) |
| Application adequacy | — | — | NOT CLAIMED — awaits S2b error budget [TBD] |

# 07 / Handoff

| Item | State |
| --- | --- |
| State | Gate v2 PASSES all declared verification-layer criteria. Evidence immutable (run dir + repo mirror). Conventions locked (energy ½, field 1, geta = 2πrA, complex bi8). R-b settled. Contour floor refuted; smoothing policy declared. |
| Open questions | (1) Loop truncation at finest mesh. (2) Solver-tolerance sweep. (3) Nonlinear manufactured case before saturation-dependent M3 conclusions. (4) Sol re-audit. (5) Adopt smoothed channel for Ampère gating (proposed: yes). |
| Next gate | Sol 5.6 re-audit (SOL-RD03-C0f) → M3 probe matrix (linked incremental AC, bias sweep) and M4 slab v6 (complex fields, 2 kHz grid, wrapped phase) on the verified extraction stack. |
| Measurements needed (Mohammed only) | None for this gate. Decision gate still needs S2b shop data. MATLAB license remains DEMO/trial — resolve before commercial use. |

# Appendix A / Sample calculations

Every number in the body traces to a hand calculation below, with values plugged explicitly. Constants: μ₀ = 4π×10⁻⁷ H/m; I = 100 A; a = 5 mm; b₁ = 20 mm.

## A.1 Coax annulus energy (§03.1, §05.2)

Substituting into Eq. (2):

## A.2 Half-annulus ∫Bx target (§03.1)

With B = Bφ(−sinθ, cosθ) over the upper half-annulus, Eq. (3):

## A.3 Relative error, finest coax energy (§05.2)

## A.4 Blind three-grid observed order (§05.3)

Fine triplet f₁ = 1.3860818e-3, f₂ = 1.3855918e-3, f₃ = 1.3841461e-3 (r = 2), in Eq. (13):

## A.5 Richardson extrapolation (§05.3)

Applying Eq. (14) with p = 1.5608:

## A.6 GCI and asymptotic-range check (§05.3)

Applying Eqs. (15)–(16):

## A.7 Manufactured-solution targets (§03.2)

Closed-form antiderivatives of Eqs. (5)–(7) over [25,85]×[15,65] mm:

## A.8 ellipke validation (§03.3)

## A.9 Loop flux target and finite-cross-section effect (§03.3)

Filament a = 30 mm, probe (20, 10) mm, via Eqs. (8) and (12):

## A.10 Harmonic ½-convention arithmetic (§05.5)

From Eq. (17) at equal numeric current:

## A.11 P4 complex target (§05.5, R-b closure)

Complex form of Eq. (3) with the phased circuit current:

## A.12 Realized refinement ratios and skin depths (§04, §07)

Realized ratios from element counts; skin depth from Eq. (18) at 5 kHz [Representative — VERIFY]:

# Appendix B / Run manifest and provenance

| Item | Value |
| --- | --- |
| Run ID | run_20260722_200912 (2026-07-22 20:09:12 America/Chicago) |
| Cache key | 358b5b996d40eedd… (SHA-256 over 9 source hashes + femm.exe + MATLAB version + params) |
| femm.exe SHA-256 | 5e83cf0899cabae2017868ddcce338e3ba4ac25d1bf8e511eac77eda2bc92a21 — EQUAL to Sol’s C0e-R record |
| MATLAB | R2026a U3 — DEMO/trial license [licensing gate] |
| Gate manifest | {stageA:true, coax:true, loop:true, femm_exe_matches_audit:true} |
| Evidence | 11 .fem/.ans · transcript 2,747 B · results.mat · report_numbers.json · 12 figures · gate_manifest.json |
| Cite | [Salari&Knupp2000],[NASA-GCI],[NASA-Loop2013],[NIST-DLMF19],[NISTIR8298],[Meeker-FEMM],[Sol 5.6 — 2026-07-22, SOL-RD03-C0e-R]; Sol rows Inbox/provisional; all run values [Measured]. |